《数字逻辑》实验报告

| 姓名 |  | 陈鹏宇， 帕肉合·帕尔哈提 |  | 年级 |  | 2020级， 2020级 |
| --- | --- | --- | --- | --- | --- | --- |
| 学号 |  | 20204227， 20201641 |  | 专业、班级 |  | 计算机科学与技术01班， 计算机科学与技术05班 |
| 实验名称 | 密码锁设计 |  |  |  |  |  |
| 实验时间 |  |  | 实验地点 |  |  |  |
| 实验成绩 |  |  | 实验性质 |  | □验证性 □设计性 □综合性 |  |
| 教师评价： □算法/实验过程正确； □源程序/实验内容提交 □程序结构/实验步骤合理； □实验结果正确； □语法、语义正确； □报告规范； 评语： 评价教师签名（电子签名）： |  |  |  |  |  |  |
| 一、实验目的 通过实验，巩固有限状态机设计方法，并能设计实现密码锁控制器。 |  |  |  |  |  |  |
| - 实验项目内容 1、设计一个密码锁电路，可以通过拨码开关设置初始密码，通过按钮来输入 密码，密码位数至少为 4 位，判断输入密码与设置的密码是否一致。 2、画出密码锁电路的状态转换图。 3、编写密码锁模块并仿真。 4、 编写顶层模块，综合、实验、生成 bit 流，下载到开发板进行验证。 |  |  |  |  |  |  |
| 三、实验设计 状态转换图 电路图 t1为密码输入，t2为按键消抖，t3为密码检测 波形图 |  |  |  |  |  |  |
| 四、实验过程或算法 源代码： /*顶层模块*/ module top_keylock( input write, reset, clk, input [7:0] sw_in, input [4:0] bt_in, output [1:0] led, output [7:0] seg, output [3:0] sel, output [9:0] out, output [3:0] state_out ); /*中间导线，用来连接各个模块*/ wire [1:0] m0,m1,m2,m3; wire [4:0] bt_in_en; /*初始密码写入模块*/ key_write t1(.write(write), .seg(seg), .sel(sel), .clk(clk), .sw_in(sw_in), .m0(m0), .m1(m1), .m2(m2), .m3(m3)); /*按键消抖模块*/ key_deb t2(.clk(clk), .reset(reset), .bt_in(bt_in), .bt_in_en(bt_in_en)); /*密码检测模块*/ key_check t3(.m0(m0), .m1(m1), .m2(m2), .m3(m3), .clk(clk), .bt_in_en(bt_in_en), .led(led), .reset(reset), .out(out), .state_out(state_out)); endmodule /*按键消抖模块*/ module key_deb( input clk, input reset, input [4:0] bt_in, output reg [4:0] bt_in_en ); parameter DURATION = 500000;//延时10ms reg [19:0] cnt; //按下计数器 reg [19:0] cnto;//未按计数器 wire key_enable; wire bt_in_float; assign key_enable = (bt_in[4] ¦ bt_in[3] ¦ bt_in[2] ¦ bt_in[1] ¦ bt_in[0]); //任意按键被按下 assign bt_in_float = ~(bt_in[4] & bt_in[3] & bt_in[2] & bt_in[1] & bt_in[0]);//按键没被按下 always @(posedge clk or posedge reset) begin if(reset) begin bt_in_en <= 5'b0; cnt <= 20'd0; cnto <= 20'd0; end else if(key_enable) begin//计数开始 cnto <= 20'd0;//抬起计数器清零 if(cnt == DURATION-1) begin cnt <= cnt; if (bt_in[4] ¦ bt_in[3] ¦ bt_in[2] ¦ bt_in[1] ¦ bt_in[0]) begin bt_in_en<=bt_in; cnt<=20'd0; end end else cnt <= cnt + 1'b1; end else if(bt_in_float) begin//计数开始 cnt <= 20'd0;//按下计数器清零 if(cnto == DURATION-1) begin cnto <= cnto; if(~(bt_in[4] & bt_in[3] & bt_in[2] & bt_in[1] & bt_in[0])) begin bt_in_en <= bt_in; cnto<=20'd0; end end else cnto <= cnto + 1'b1; end else begin cnt <= 20'd0; cnto <= 20'd0; end end Endmodule `timescale 1ns / 1ps /*密码检测模块*/ module key_check(m0, m1, m2, m3, clk, bt_in_en, led, reset, state_out, out); output reg [1:0] led;//输出解锁成功与否 output wire [9:0] out; //输出check2和按键状态 output wire [3:0] state_out;//输出当前状态 input clk; //时钟信号 input reset; //重置状态 input [1:0] m0, m1, m2, m3;//memery中的密码 input [4:0] bt_in_en; //通过button输入密码，其中bt[0-3]分别表示00，01，10，11 reg [3:0] state; //当前状态机状态 /*led灯状态*/ parameter led_begin = 2'b00; parameter led_true = 2'b10; parameter led_false = 2'b01; /*按键输入状态*/ parameter put0 = 5'b00001; parameter put1 = 5'b00010; parameter put2 = 5'b00100; parameter put3 = 5'b01000; parameter putok = 5'b10000; parameter putd = 5'b00000; /*实际的检测顺序，从左至右(按实际场景，应从左至右检测密码所以从m3开始)*/ reg [4:0] check1 = (m3 == 3) ? put3: (m3 == 2) ? put2: (m3 == 1) ? put1: put0; reg [4:0] check2 = (m2 == 3) ? put3: (m2 == 2) ? put2: (m2 == 1) ? put1: put0; reg [4:0] check3 = (m1 == 3) ? put3: (m1 == 2) ? put2: (m1 == 1) ? put1: put0; reg [4:0] check4 = (m0 == 3) ? put3: (m0 == 2) ? put2: (m0 == 1) ? put1: put0; reg [4:0] checkok = putok; /*状态机状态*/ parameter s0 = 4'b0000; parameter s1 = 4'b0001; parameter s2 = 4'b0010; parameter s3 = 4'b0011; parameter s4 = 4'b0100; parameter s5 = 4'b0101; parameter s6 = 4'b0110; parameter s7 = 4'b0111; parameter s8 = 4'b1000; parameter s9 = 4'b1001; parameter s10 = 4'b1010; parameter s11 = 4'b1011; always @(posedge clk) if (reset) // 重置 begin led <= led_false; state <= s0; end else case (state) // mealy有限状态机 s0: begin if (bt_in_en == check1) begin led <= led_begin; state <= s1; end else if (bt_in_en == putd) begin led <= led_begin; state <= s0; end else begin led <= led_begin; state <= s10; end end s1: begin if (bt_in_en == putd) begin led <= led_begin; state <= s2; end else if (bt_in_en == check1) begin led <= led_begin; state <= s1; end else begin led <= led_begin; state <= s10; end end s2: begin if (bt_in_en == check2) begin led <= led_begin; state <= s3; end else if (bt_in_en == putd) begin led <= led_begin; state <= s2; end else begin led <= led_begin; state <= s10; end end s3: begin if (bt_in_en == putd) begin led <= led_begin; state <= s4; end else if (bt_in_en == check2) begin led <= led_begin; state <= s3; end else begin led <= led_begin; state <= s10; end end s4: begin if (bt_in_en == check3) begin led <= led_begin; state <= s5; end else if (bt_in_en == putd) begin led <= led_begin; state <= s4; end else begin led <= led_begin; state <= s10; end end s5: begin if (bt_in_en == putd) begin led <= led_begin; state <= s6; end else if (bt_in_en == check3) begin led <= led_begin; state <= s5; end else begin led <= led_begin; state <= s10; end end s6: begin if (bt_in_en == check4) begin led <= led_begin; state <= s7; end else if (bt_in_en == putd) begin led <= led_begin; state <= s6; end else begin led <= led_begin; state <= s10; end end s7: begin if (bt_in_en == putd) begin led <= led_begin; state <= s8; end else if (bt_in_en == check4) begin led <= led_begin; state <= s7; end else begin led <= led_begin; state <= s10; end end s8: begin if (bt_in_en == checkok) begin led <= led_begin; state <= s9; end else if (bt_in_en == putd) begin led <= led_begin; state <= s8; end else begin led <= led_begin; state <= s10; end end s9: begin if (bt_in_en == putd) begin led <= led_true; state <= s9; end else if (bt_in_en == checkok) begin led <= led_begin; state <= s9; end else begin led <= led_begin; state <= s10; end end s10: begin if (bt_in_en == putok) begin led <= led_begin; state <= s11; end else begin led <= led_begin; state <= s10; end end s11: begin if (bt_in_en == putd) begin led <= led_false; state <= s11; end else if (bt_in_en == checkok) begin led <= led_begin; state <= s11; end else begin led <= led_begin; state <= s10; end end endcase assign state_out = state; assign out = {check3,bt_in_en}; endmodule `timescale 1ns / 1ps /*初始密码写入模块*/ module key_write(write, clk, seg, sel, sw_in, m0, m1, m2, m3); output reg [7:0] seg; output reg [3:0] sel; //七段数码管显示和选通 output wire [1:0] m0, m1, m2, m3; input clk; //时钟信号 input write; //写数据 input [7:0] sw_in; //初始密码拨键输入 reg [2:0] dispsel; //选通 reg [1:0] mem [3:0]; //存储write_btword的数据 reg [1:0] dpdat; //tmp data reg [19:0] count = 0; always @(posedge clk or posedge write) begin if(write) begin mem[0] <= sw_in[1:0]; mem[1] <= sw_in[3:2]; mem[2] <= sw_in[5:4]; mem[3] <= sw_in[7:6]; end else begin end end assign m0 = mem[0], m1 = mem[1], m2 = mem[2], m3 = mem[3]; /*七段数码管显示模块*/ always@(posedge clk) //分时复用 begin count <= count + 1; if(count == 50000) begin count <= 0; dispsel <= dispsel + 1; if(dispsel == 3) dispsel <= 0; end end always@(posedge clk)//分配选通 case(dispsel) 0: begin sel = 14; dpdat = mem[0]; end 1: begin sel = 13; dpdat = mem[1]; end 2: begin sel = 11; dpdat = mem[2]; end 3: begin sel = 7 ; dpdat = mem[3]; end default: begin sel = 7; dpdat = 15; end endcase always@(posedge clk)//数码管显示 begin seg[0] <= 1; case(dpdat) 2'b00 : seg[7:1] <= 7'b0000001; 2'b01 : seg[7:1] <= 7'b1001111; 2'b10 : seg[7:1] <= 7'b0010010; 2'b11 : seg[7:1] <= 7'b0000110; default: seg[7:1] <= 7'b1111111; endcase end endmodule 仿真文件： `timescale 1ns / 1ps module sim_lock; reg write = 1, reset = 0, clk = 0; reg [7:0] sw_in = 8'b11100100; //3210 reg [4:0] bt_in = 5'b00000; wire [1:0] led; wire [3:0] state_out; default_password u(write,reset,clk,bt_in,sw_in,led,state_out); always #50 clk = ~clk; initial begin #100 bt_in = 5'b01000; //3 #100 bt_in = 5'b00000; #100 bt_in = 5'b00100; //2 #100 bt_in = 5'b00000; #100 bt_in = 5'b00010; //1 #100 bt_in = 5'b00000; #100 bt_in = 5'b00001; //0 #100 bt_in = 5'b00000; #100 bt_in = 5'b10000; //ok end endmodule |  |  |  |  |  |  |
| 五、实验过程中遇到的问题及解决情况 首先是按键防抖模块，对于教程中的代码一直不能防抖成功，所以只能在网上搜寻相关模块，如下图，测试多个模块后才能成功防抖。 如果在两个always语句中对同一个变量赋值，会出现multi_driven net on pin的错误，需要修改或删去其中一个赋值。 出现以下错误，错误原因尚不明确，但在xdc文件中，加入 set_property CLOCK_DEDICATED_ROUTE FALSE [get_nets clk] set_property CLOCK_DEDICATED_ROUTE FALSE [get_nets reset_IBUF] set_property CLOCK_DEDICATED_ROUTE FALSE [get_nets check_IBUF] 便可修复。 在状态机设计时，会出现状态不动的情况，只需加入停滞状态即可修复，即每个检测密码状态之后加上一个没有按键输入的状态，在开发板中显示为按下和抬起按键为不同状态。 编码过程中还出现了一些语法错误，便不在此赘述了。 |  |  |  |  |  |  |
| 六、实验结果及分析和（或）源程序调试过程 为了使代码看起来简洁，逻辑更加清晰，采用多模块的编码方式。分为初始密码输入模块，通过拨键开关输入，输入完成后密码将显示在七段数码管上；密码检测模块，通过按键开关输入，其中输入完后需要按确认键才能显示开锁与否；按键防抖模块。 I/O接口： Input ：sw[7:0]从左到右每两位输入一个密码数字 sw[15]write接口，高电平时能修改密码 sw[14]reset接口，高电平时重置状态 bt[3:0]输入密码接口，分别对应输入3，2，1，0 bt[4]确认密码接口，按下后将显示开锁与否 Output:led[15:14]开锁与否接口，10为成功，01为失败 led[13:10]状态输出接口，将输出当前状态 led[9:5]自定义check接口，用来调试程序，这里为check2 led[4:0]按键输出接口，按下按键对应led便会高电平 七段数码管，显示默认密码 调试： 通过改变自定义接口的绑定，通过led灯的输出，来判断某个变量是否正确。 实验结果（演示视频将放在压缩包中）： 刚开始上板时，默认密码是0000 通过write和sw，来修改密码为3210 现来展示检测密码，每按下或抬起按键为不同状态，led能显示按下的 是哪一个按键和当前的状态 正确输入四位密码后，按下确认键，即密码解锁，led显示为10 若错误密码，或密码长度不正确，按下确认键后都会失败，led为01（注：必须按下ok键后才能显示状态） - 小组分工情况说明 陈鹏宇：自主完成实验，完成实验报告 帕肉合帕尔哈提：自主完成实验 |  |  |  |  |  |  |